Document Identification: SA40-0001

Revision: 10

Technical Sales Specification

Offshore Drilling

Revision History

| Rev. | Reason for issue | Date |
| --- | --- | --- |
| 10 |  |  |
| 09 | Table 3-1 Functional requirements NORSOK D-001 Table 8-1 – Added note on components inside controlled environments Table 8-3 and 8-4 accelerations changed Table 12-1 Some additional applications added. Chapter 4 Clarified conditions for replacements and upgrades Chapter 5 API monogram rewritten Chapter 6 CE Marking – adapted to new wording from EU Chapter 7.6 Hazardous area classification Chapter 11 Paint standard reference ISO 12944 replacing NORSOK M501 Chapter 12 Welding standards to include API requirements when applicable. Chapter 15.3 Low smoke and halogen for all cables. Clarification of non-compliant cable handling Chapter 15.7 Protective earthing though armor and through gland described Chapter 16.4 Replaced alarm standards Various clarifications and simplifications | January 15, 2025 |
| 08 | Chapter 1 Included statements relating to PO/ contract Chapter 3 Minor corrections table 3-1 Chapter 4 Minor corrections table 4-1 Chapter 4 Exceptions to be detailed with bid Chapter 5, API compliance re-written Chapter 6, CE marking re-written Section 7.1-7.5 minor corrections Section 7.6 Ignition Hazard Assessment ISO 80079 is included Chapter 8 Included general statement about design criteria Section 8.3 New motion tables Section 8.6 Deleted statement about earthquake in Norway. Chapter 9 Mechanical design moved from chapter 10 to chapter 9 Chapter10 Reliability Centered Maintenance, New chapter Section 11.3 Preservation, minor corrections Chapter 12 Materials and Welding, minor corrections Chapter 14 Fluid power, minor corrections Chapter 15 Electrical control and power systems, minor corrections Chapter 16. Control systems, minor corrections Section 16.7 Cyber security re-written with reference to recognized standards Section 18.2 SMDL first issue 2 weeks after order Section 18.2 Turnaround time 10 working days for document review Section 18.5 Identification of products- new section Chapter 19 New references included | May 18, 2020 |
| 07 | Document table 18-1 updated. EU directives, references corrected ABS MODU and IECEx included in table 3-1 Verification of lifting equipment table 3-1. Extent of third party to be agreed. Chapter 15 and 16 title changed Cyber security, section 16.7 is added Updated structural design references Rig motion table 8-3, added note. Added data to table 14-1 and 14.2 14.5 Deleted brand names for hydraulic oil. (Ref user manual). Changed IP class table 15-1 Cable glands for customer’s cables, 15 SMDL 18.2 text re-phrased for clarity. Some minor clerical corrections. | February 15, 2018 |

Table of Contents

1	Introduction	4

2	Scope	4

3	Principal Regulations – Selectable Categories	5

4	Normative References	7

5	API Monogram	8

6	CE Marking	8

7	Safety	9

8	Design Basis	11

9	Structural and Mechanical Design	14

10	Reliability Centered Maintenance	15

11	Surface Protection and Preservation	16

12	Materials and Welding	18

13	Piping	19

14	Fluid Power Systems	20

15	Electric Instrumentation and Power Systems	22

16	Control systems, HMI and networks	25

17	Testing	27

18	Documentation and Marking of Equipment	28

19	Appendix A – Reference Documents	32

# Introduction

This document is a technical specification of HMH drilling products describing the basic quality which is built into the products by default.

This specification is intended to meet the demands that are commonly expected within the offshore industry. However, any and all warranties and/or guarantees in equity or at law are waived.

This specification shall be read in conjunction with the relevant product specification(s), HMH document code SA04, the general arrangement drawing and HMH’s quotation letter, which give additional information.

Apart from this, there is no specific reference to any other HMH standards. The quality is based on internationally recognized standards which are implemented in HMH’s internal procedures and governing documents, to the extent applicable and/or desired.

This specification can be used as a contract basis without any additional requirements, or as a platform for further negotiations, which can result in a revised offer.

Principal regulations, as described in chapter 3, must be clearly identified in the contract. Although the technical quality is standardized, and may need only minor amendments, there may be various administrative requirements that have an impact on cost and delivery time (e.g. documentation and approvals).

Any options mentioned in this document shall be priced separately, and are consequently excluded from the scope of work and price.

This document will be revised and updated on a regular basis, to ensure the technical quality is compliant with relevant regulations and market demands.

# Scope

With reference to the HMH product catalogue, the following groups of products are included:

Structures and Traveling Equipment

Drilling, Make and Break

Handling Equipment and Cranes

Motion Compensation Systems

Control and Automation

Drilling Fluid Processing and High Pressure Manifolds

Risers

Not included in this specification: (Not exhaustive)

Products intended for land-based drilling,

Products for oil and gas producing systems

Products of other brands or make than HMH

Activities belonging to the shipbuilding scope or plant engineering

Spare parts for products built to earlier standards, unless specially agreed.

# Principal Regulations – Selectable Categories

The products can be delivered in compliance with the principal regulations and codes listed below. The selected option must be stipulated in the contract.

Table 3-1 - Principal regulations

| Regulation Category | Drilling Facility or Unit Category | Activities / Deliveries | Compliance |
| --- | --- | --- | --- |
| 1 – Basic | Facilities with no special territorial requirements, or mobile units with a basic maritime class notation, not including the drilling plant. | Conformity is declared in Affidavit of Compliance (Works Certificate) issued by HMH | Products are compliant with the default standards noted in Table 4-1. ATEX or IECEx compliant- as applicable and reflected in the price. |
| 2 - DNV with DRILL notation | Mobile unit where certification of the drilling plant by DNV is required. Also including well intervention units with WELL notation | Products are certified by DNV as stipulated in DNVGL-OS- E101. Certification is priced separately. | Products are compliant with the default standards noted in Table 4-1. |
| 3 - DNV with DRILL-N notation | Mobile unit registered in a national shipping register and dedicated for operating on the Norwegian Continental Shelf and where certification of the drilling plant is required. Also including well intervention units with WELL-N notation. | Products are certified by DNV as stipulated in DNVGL –OS- E101 and relevant Norwegian regulations. Certification is priced separately. | Products are compliant with the default standards noted in Table 4-1 and the following: - NORSOK R-002 for lifting equipment - NORSOK S-002 as applicable and reasonable - Norwegian FME regulations for electrical installations, if registered with Norwegian flag. |
| 4 - ABS with CDS notation. (Maltese-cross-CDS) | Mobile unit where certification of the drilling plant by American Bureau of Shipping is required. | Products are certified by ABS as stipulated in “Guide for Classification of Drilling System” (CDS guide). Certification is priced separately. | Products are compliant with the default standards noted in Table 4-1. And applicable parts of ABS MODU rules |
| 5 - Norwegian Fixed installations | Drilling facilities permanently placed at the Norwegian continental shelf. (including buoyant vessels at permanent position) | Equipment shall be CE marked and HMH will issue a declaration of conformity or declaration of incorporation. Verification of lifting equipment according to NORSOK R-002. Extent of third party verification to be agreed with client and priced separately. | Products are compliant with the default standards noted in Table 4-1 and the following: - CE marking (Ref. chapter 6) - NORSOK R-002 for lifting equipment - NORSOK S-002 as applicable and reasonable for drilling products. - NORSOK D-001 as applicable for drilling products. |
| 6 - Other | Other rules, regulations or requirements. HMH is a highly flexible vendor of drilling equipment and has long experience in adapting to different regulations and requirements around the world. | Assessment of local regulations. Activities and deliveries required in regulations will be planned. Certification to local regulations when applicable. Design changes may be required | Products are compliant with the default standards noted in Table 4-1 and amendments shall be agreed with client or governing Body. Examples may include: Canadian, Russian and UK regulations. Lloyds, BV, and other classification rules. |

# Normative References

Within the provisions of the principal regulations, the internationally recognized standards listed in table 4-1 are used as our main reference.

Within the applicable area as noted in the table, our products shall meet all relevant requirements in these standards, unless otherwise detailed in the product specification provided with the bid.

Standards of limited application is referred in the document. These are not listed in table 4-1, and apply as explained in the text. The complete overview of references is given in appendix A.

For new rig projects, latest edition effective at time of tender shall apply, unless otherwise agreed.

For replacement of equipment, rules and editions from the time of original build applies, however HMH may choose to use current editions as applicable. (E.g. for stocked items). Equipment upgrades may be subject to re-certification as applicable. Exceptions to be clarified.

Table 4-1 - Application areas for the main normative references.

| Standard document | Areas for application |
| --- | --- |
| ABS Guide for Classification of Drilling System (CDS) | Technical requirements are implemented by default for all equipment as relevant. Certification when selected. |
| API STD 2RD | Drilling risers – general dynamic risers for floating production systems |
| API Spec 4F | Derricks, compensator structure, load criteria for design of fingerboards. |
| API Spec 5L, ISO 3183 | Drilling risers – main joints |
| API Spec 6A | Choke and kill manifold, Standpipe manifold, Gate valves |
| API Spec 7K | Mud pumps, drawwork and rotary table. Mud hoses |
| API Spec 8C | Hoisting equipment; |
| API Spec 16A, ISO 13533 | Drilling risers – quality control |
| API Spec 16C | Choke & kill manifold, Choke control system. Risers: Choke & kill lines |
| API Spec 16F | Drilling risers – general for the whole scope. Sec. 13 for buoyancy |
| API RP 16Q and DNV F201 | Marine drilling riser systems:design, selection, operation, maintenance |
| API Spec 16R, ISO 13625 | Drilling risers – couplings |
| API RP 17B, ISO 13628-11 | Drilling risers – flexible pipe. Supplements 13628-2 and 13628-10. |
| API Spec 17D | Drilling risers – lifting equipment |
| API Standard 53 | Well Control Equipment Systems for Drilling Wells |
| ASME B31.3 | Piping design, fabrication and testing. |
| ASME BPVC section VIII | Pressure design and analysis, risers and pressure vessels. |
| DNVGL-OS- E101 Drilling Facilities | Technical requirements are implemented by default for all equipment as relevant. Certification when selected |
| Eurocode 3 | Structural design |
| IEC 60079 | Explosive atmospheres. Equipment. Relevant parts |
| IEC 61131-3 | Programmable controllers. Part 3: Programming languages |
| IEC 61158 | Industrial communications networks. Fieldbus specification |
| IEC 61892 | Mobile and fixed offshore units. Electrical installations |
| NACE MR 0175/ISO 15156 | H2S corrosion. Risers, Choke and Kill, Cement and Standpipe manifold |
| NORSOK Z-006 | Preservation |
| NORSOK Z-018 | Documentation of equipment |

# API Monogram

The following equipment may be supplied with API monogram:

Rotary tables API 7K-0551

Mud pump components API 7K-0551

Drawwork components API 7K-0551

Power swivels (Top drive) PSL 1 API 8C-0351

Travelling blocks PSL 1 API 8C-0351

The implementation of API monogram license for other product types is continuously evaluated by HMH, based on market demand.Current monogram certifications are published at the “API composite list” under company names MHWirth or HMH.

# CE Marking

European EU directives/regulations shall apply when mandatory

Equipment for non-movable installations within the EU/EEA/UK is CE marked by HMH.

CE marking is not applicable for territories outside the EU/EEA/UK, for seagoing vessels and mobile offshore units. HMH will as normal not CE mark such equipment, but the technical requirements of the directives/regulations may be implemented as an option to any customer upon agreement.

Upon delivery of the equipment, an EC/EU Declaration of Conformity comprehending all relevant directives and harmonized standards is issued by HMH.

Technical files and declarations of components are kept by HMH or notified body as mandated by the relevant EU directive.

Partly complete equipment is not CE marked. A declaration of incorporation is issued as mandated in the relevant EU directive.

The following EU directives are included, when relevant:

Equipment for explosive atmospheres 2014/34/EU (ATEX)

Machinery Directive 2006/42/EC (MD)

Pressure Equipment Directive 2014/68/ EU (PED)

Electromagnetic Compatibility Directive 2014/30/EU (EMC)

Low Voltage Directive 2014/35/EU (LVD)

Note: High pressure manifolds for well control are exempted from the Pressure Equipment Directive.

# Safety

## Risk Assessment and Risk Reduction

A risk assessment is performed on all HMH products with the objective of reducing risk to the lowest practicable level to prevent harm to personnel, property and environment.

To achieve safety in the design of HMH products, general principles and methodology for risk assessment and risk reduction is based on and performed according to ISO 12100

The preferred hazard analysis method used is FMEA (Failure Mode and Effect Analysis), with the objective to make our equipment inherently safe by design, as far as reasonably practicable

Information for safe use is integrated in the user manual, including recommended working practices and warnings of residual risk

## Safety Design

For safety of machinery- electrical equipment of machines, IEC 60204-1 is applied

For safety of hydraulic and pneumatic systems ISO 4413 and ISO 4414 is applied

Emergency stop system is in accordance with ISO 13850 with minimum ISO13849-1 Performance Level d

Equipment for overhead installation is secured according to Dropped Objects Awareness and Prevention” (DROPS) and amendments by the Norwegian “Working Together for Safety” organization. The best practices of these two organizations are followed.

## Access

Access ways for operating the equipment are in accordance with ISO 14122-1, which is the main reference

Access for maintenance and emergency of cranes and other equipment is designed according to EN 13586

## Ergonomics

Control panels and other control devices for manual operation of equipment are placed in safe locations.

Ergonomic design of drilling control rooms and crane cabins is according to ISO 11064

## Noise

CE marked equipment is supplied with information about noise emission in accordance with the Machinery Directive 2006/42/EC.

Information about noise from non-CE marked equipment can be provided by HMH as an option

Noise measurements, when required, are typically performed according to ISO 3746 or ISO 9614-2

Modifications or additions to meet the noise level for a room or area are beyond the scope of this specification.

## Hazardous Area Classification

Equipment is delivered according to the hazardous area classification specified by the customer in each project, and is detailed in the equipment specifications submitted with the bid from HMH.

Electrical and electronic equipment for hazardous areas is based on IEC 60079, and certified to ATEX or IECEx, as applicable.

For non-electrical equipment in hazardous area ignition hazard assessment (IHA) is performed according to EN 1127-1 and ISO 80079.  This is part of HMH basic design risk assessment.

The technical file, including the IHA, is reported according to the ATEX directive and filed with the Notified Body as mandated by legislation or contract.

All deliveries are certified for gas group IIB and temperature class T3 (≤ 200 °C) or stricter.

# Design Basis

Design criteria given in this specification are basis for pricing, schedule and weight estimates, unless other criteria are expressly specified in the order by customer. Change of criteria after contract shall be processed as variation order according to applicable terms and conditions.

## Lifetime and Operational Conditions

The equipment is designed and built to operate in marine offshore conditions

Primary structural parts of the equipment are designed for a fatigue life of 20 years operation in typical drilling program cycles. Normal wear and tear shall be expected.

Recommended services and maintenance programs must be followed

Design basis for drilling risers is established case by case, beyond this specification.

## Temperature

The equipment is designed to operate at ambient temperatures given in Table 8-1 below, using lubricants and hydraulic fluids that suit the local seasonal climate, as advised by HMH.

Equipment in contact with drill fluids or well fluids is designed for fluid temperatures as provided.

Table 8-1 - Ambient and design temperature for drilling equipment

| Equipment | Ambient temperature (°C) | Drill or well fluids equipment design temperature (°C) |
| --- | --- | --- |
| Standard outdoor | From minus 20 to plus 45 | NA |
| Installed indoors *) | From 0 to plus 45 | NA |
| Choke/kill, cement and standpipe manifold | From minus 29 to plus 45 | From minus 29 to plus 121 |
| HPHT (high pressure high temperature) | From minus 29 to plus 45 | Minus 29 to plus 177 Optional for choke and kill |
| Mud reconditioning equipment | Indoor or outdoor as above | Maximum 70 |
| Drilling risers |  | Minus 29 to plus 82 |

*) Does not apply to components located inside controlled environment

## Wind Loads

If no wind information is specified in the contract, the wind speeds shown in Table 8-2 are used as a basis.

Table 8-2 - Wind speed

| Condition | Case | U(10m,3s) = wind speed at 10 m above sea level, 3 sec gust | U(10, 3600s) = wind speed at 10 m above sea level, 1 hour mean |
| --- | --- | --- | --- |
| All | Operation | 33 m/s | 26 m/s |
|  | Riser, BOP handling | 21 m/s (typical ⅔ of the operation wind force) | 17 m/s (typical ⅔ of the operation wind force) |
|  | Survival, 100 years | 45 m/s* | 36 m/s* |
| All | Damage, 27 degrees | 0 m/s (included in the damage heel) | 0 m/s (included in the damage heel) |

* The survival wind speed is based on recommendations in NORSOK N-003 for the Norwegian continental shelf.

The wind profiles and gust are following the formulas given in DNV-RP-C205.The average gust period used for designing the equipment is 3 seconds.

## Rig Motion Loads

If the rig motion accelerations are not given, the accelerations given in table 8-3 and 8-4 are used. All values are given as single amplitude and as a fraction of gravity (g = 9.81 m/s2).

Table 8-3 Rig motions for semi-submersible

| Motion condition* | Case | Horizontal acceleration |  | Heave |
| --- | --- | --- | --- | --- |
|  |  | At Drill floor | Increases upwards |  |
| Normal | Operation | 0.10 g | 0.0021 g/m | 0.07 g |
|  | Riser and BOP handling | 0.07 g | 0.0014 g/m | 0.05 g |
|  | Survival, 100 years | 0.15 g | 0.0028 g/m | 0.15 g |
| High | Operation | 0.15 g | 0.0022 g/m | 0.10 g |
|  | Riser and BOP handling | 0.10 g | 0.00147 g/m | 0.07 g |
|  | Survival, 100 years | 0.25 g | 0.0051 g/m | 0.20 g |
| Damage | Accidental heel, 27 degrees | 0.45 g | NA | 0.89 g |

Table 8-4 Rig motions for drill-ship

| Case | Horizontal acceleration |  | Heave |
| --- | --- | --- | --- |
|  | At Drill floor | Increases upwards |  |
| Operation | 0.15 g | 0.0038 g/m | 0.15 g |
| Riser and BOP handling | 0.10 g | 0.00253 g/m | 0.10 g |
| Survival, 100 years | 0.30 g | 0.0046 g/m | 0.20 g |
| Accidental heel, 27 degrees | 0.45 g | NA | 0.89 g |

Accelerations due to wave action on fixed installations are not considered.

## Snow and Ice

This shall be considered for cranes working under exceptional conditions, and equipment which is particularly sensitive to such effects. Heat tracing devices are not included, but may be supplied as an option.

## Earthquake

Equipment for installations on the seabed may be affected by accelerations due to earthquake.

As this type of loads can vary considerably, the seismic spectra for each specific area shall be specified in the order by the customer. HMH will consider the effect upon the equipment.

Necessary reinforcement of the equipment due to earthquake shall be priced separately.

## Accidental Loads

Accidental heel of floating rig or vessel in damaged condition and dropped object are included in the design of the actual equipment.

Explosion loads are not likely to affect dimensioning of equipment, and is normally not accounted for in the design, except for derrick cladding, cabins and cabinets.  It will then be part of the plant design, outside the scope of this specification.

# Structural and Mechanical Design

The structural integrity of the equipment is checked with respect to the possible modes of failure:

- Yielding

  - In accordance to EN1993-1 Eurocode 3.

- Buckling

  - Frame/beam structures are in accordance to EN1993-1 Eurocode 3.

  - Stiffened and unstiffened plated structures in accordance to EN1993-1 Eurocode 3 or DNV-RP-C201 Buckling strength of plated structures.

- Fatigue

Generally, fatigue capacity is in accordance to DNV-RP-C203 Fatigue design of offshore steel structures.

The structural integrity assessment of the equipment is based on load cases, load- and resistance-factors in accordance with the equipment specific standards listed in Table 19-1.

The most unfavorable position, direction and combination of loads are considered.

The stiffness of the structure is also checked in order to ensure acceptable displacements and vibrations.

Mechanical components for lifting and handling equipment appliances are designed according to DNV-ST-0378. Main hoisting draw works are according to API 7K.

Gears, bearings, seals and other mechanical components, which are generally according to vendor’s standard, shall be evaluated and selected by HMH as part of the design process.

All selection of components shall be the responsibility of HMH.

# Reliability Centered Maintenance

Through HMH’s Reliability Centered Maintenance (RCM) service, customized reliability and maintenance analyses are outlined to define a maintenance management regime focusing on cost-effectiveness and risk reduction. This drives effective proactive- and condition-based maintenance (CBM) and prompts the extension of the renewal classification beyond the normal 5 years interval.

To prepare the equipment for condition based maintenance, a qualitative RCM analysis will be performed as part of our design, and a special maintenance manual will be developed for this purpose. This is an option that is available for critical equipment, as detailed in the particular product specification.

RCM analysis is performed in accordance with the following standards

SAE JA1011-Evaluation Criteria for RCM Process

SAE JA1012 A Guide to the Reliability Centered Maintenance (RCM) Standard

IEC 60300 Dependability Management part 3-11 Application Guide -Reliability Centered Maintenance

Additional services offered by HMH related to implementation and monitoring during operation are beyond the scope of this specification

# Surface Protection and Preservation

## Paint System

Equipment structures and main components HMHexposed to marine atmosphere are painted according to HMH standards, conforming to ISO 12944-9 class CX. (corresponding to Norsok system 1B)

HMHVendor components may be coated according to vendor’s standard based on ISO 12944-5 or equivalent, at the discretion of HMH.

Drilling riser elements for immersed service are painted to ISO 12944-9 class Im4 (corresponding to Norsok system 7.)

Isocyanate based coatings are not used by HMH.

The internals of hydraulic oil tanks and dry bulk tanks are considered non-corrosive during operation. A coat of primer is applied for temporary protection before use.

Stainless steel surfaces are normally not painted, except insulated pipe and vessels, some architectural items, and hydraulic p-lines in drilling riser system. These items are painted with 2 coats epoxy system.

Parts of the mud and cement system that are exposed to erosion may be supplied unpainted and designed with adequate allowance for corrosion and erosion

## Galvanizing

Equipment suitable for dipping may be hot dip galvanized according to ISO1461, as deemed practical by HMH

Typical outfitting structures like handrails, platforms, and bolted derrick members can be hot dip galvanized

Unless specifically requested and compensated by the customer, galvanized surfaces are not painted.

The following items are normally not hot dip galvanized due to risk of damage:

Elements with critical dimension tolerances

Elements of high strength steel or complex nodes with thick material

Riser elements

## Preservation

Preservation is defined as temporary protection prior to commissioning and initial operation.

The preservation of equipment is based on the principles of NORSOK Standard Z-006, as relevant for supplier’s phase, which includes:

Establish procedure for initial preservation, as applicable to scope of supply.

Perform initial preservation before delivery to customer.

Prepare instructions or checklists for maintenance preservation to be continued by customer or others after delivery from manufacturer.

Issue the preservation records, checklists and labels to document the preservation status as supplied by HMH, and to enable continuation by others.

The preservation is performed according to standardized procedures, developed for overseas transport and storage at construction site for a total period of 12 months after delivery. Longer preservation time shall be mutually agreed between HMH and the customer

The preservation records and checklists for the customer actions after delivery, are given in the Installation, Operation and Maintenance, documentation. Copies shall also be enclosed with the shipment, as part of the dispatch dossier.

# Materials and Welding

## Load Bearing Materials

Materials for the main load bearing mechanical, structural and pressurized parts of the equipment are selected within the provisions of the principal design codes and standards listed in table 4-1.

Impact toughness is tested for the lowest design temperature, as stipulated in the applicable code. Secondary steel is normally not impact tested.

## Other Materials

Table 12-1  Typical materials for miscellaneous applications.  Not governed by design code.

| Equipment | Material |
| --- | --- |
| Fasteners, diameter < 12 mm | Stainless steel 316 (A4-70 or A4-80) |
| Fasteners, diameter ≥ 12 mm | Low alloy steel. Hot dip galvanized |
| Fasteners for rises | Low alloy steel ASTM 320 Xylan coated |
| Socket cap screws in machinery | Low alloy steel, black finished, to be painted or preserved |
| Fluid power tube and connectors | Stainless steel 316 |
| Hose fittings | Zinc plated and preserved |
| Cable bridges | Stainless steel 316 |
| Outdoor cabinets | Stainless steel 316 |
| Cabinets in weather protected field area | Stainless steel 304 |
| Junction boxes | Stainless steel 316 or glass fiber reinforced plastic, GRP |
| Cable glands | Nickel plated brass or stainless steel 316 (optional), Plastic for GRP enclosures |
| Hydraulic manifold blocks | Ductile iron, steel or aluminum (marine grade) |
| Hydraulic reservoirs | Painted carbon steel or stainless steel 316 (optional) |
| Indoor control cabinets dry room | Painted carbon steel or GRP/stainless |
| Butterfly valves - for LP mud and bulk | Ductile iron body, Epoxy coated disc, Stainless steel shaft |
| Pneumatic actuators for butterfly valves | Aluminum or plastic base with clear resin lid solenoid |
| Miscellaneous outfitting: | Steel or aluminum (marine grade) |
| Fasteners for aluminum, any size | Stainless steel 316 |
| Corrosion resistant ring groove inlay | Inconel 625 is standard for choke and kill and standpipe System |
| Corrosion resistant seat pockets | Inconel 625 is standard for choke and kill manifold |
| Miscellaneous vendor components | Vendor standard, as deemed fit for purpose by HMH |

Where alternatives are given, the selection is at HMH’s discretion, unless otherwise expressly agreed.

## Dissimilar Materials

Risk of galvanic corrosion are adhered to for joints between dissimilar materials.

For bolted joints that need a firm connection, the use of insulation gaskets is avoided. Methods of protection for atmospheric conditions are given in HMH’s internal design manuals.

For submerged conditions and sweater pipe systems special solutions are implemented.

## Welding

Structural and mechanical components of drilling equipment are welded according to DNV-OS-E101 and normative references given therein.

For products with API monogram, any additional requirements given in the applicable API product specification shall also be met (table 4-1).

Weld procedures are tested and qualified to verify mechanical properties, including low temperature toughness. The qualification is subject to survey by DNV or ABS, when mandatory, and otherwise by workshop’s certified welding engineer or HMH

Procedures shall be valid as long as they meet applicable norms, and are not re-qualified for a project.

Nondestructive examination and testing is performed to the extent and standards as stipulated by the design code and DNV and ABS

Pipe welding, inspection and testing is according to ASME B31.3 or equivalent European standard recognized by DNV and ABS.

Pressure vessel welding inspection and testing is according to ASME VIII or equivalent European standard recognized by DNV and ABS

# Piping

Pressure piping within the equipment is designed, built and tested according to ASME B31.3 as the principal piping code.

Drilling riser piping is designed, built and tested according to relevant API, DNV and NACE standards listed in table 4-1.

Suction pipes and drain pipes are designed and built according to internal HMH procedures.

Small bore tubing for fluid power are of metric size, detailed in pipe class sheets developed by HMH. The pressure capacity is designed according to ASME.

All pressure piping is pressure tested before delivery (not tank return and leak oil piping).

The piping within the equipment is an intrinsic part of the product design and spare part hierarchy, hence it is not to be customized.

HMH can supply an optional interface kit to the customer’s piping system.

# Fluid Power Systems

## General

The term fluid power comprises both the hydraulic and the pneumatic system.

The fluid power symbols and circuit diagrams are based on ISO 1219, part 1 and 2.

The fluid power interfaces towards the customer will be specified in the product specifications.

## Interface Performance Data

The fluid power interface performance dataset is provided in the product specification.
Typical examples are shown in the tables below.

Table 14-1 - Typical utility and interface requirement for hydraulic power supplied by the customer

| Description | Technical data |
| --- | --- |
| Power pack type | Constant Pressure supply |
| Fluid type compatibility | Hydraulic mineral oil ISO 6743-4/ISO-L-HM */** *ISO-L-HM is the minimum required additive class. ISO-L-HV: recommended for a wider operational temperature range of the equipment. ** Environmentally acceptable fluid compatibility – consult HMH. |
| Minimum fluid start up temperature | -20 °C |
| Maximum fluid temperature | 60 °C (140 °F) |
| Optimum operational viscosity range | 16-150 cSt |
| Minimum operational viscosity | 12 cSt (short term) |
| Maximum operational viscosity | 400 cSt (short term) |
| Permissible upper operational contamination limit | According to ISO 4406 code - / 15 / 13 |
| Permissible upper operational water content | 500 ppm |
| Maximum consumption flow | 180 l/min [47,6 US gpm) @190 bar (57 kW) |
| Maximum working pressure | 210 bar (3045 psi) |
| Minimum pressure required at max flow | 140 bar (2030 psi) |
| Minimum pressure required at < 30% of max flow | 190 bar (2755 psi) |
| Maximum tank return flow | 240 l/min [63,4 US gpm) |
| Max permissible pressure at hook up interface, tank return@max tank return flow | 5 bar (72 psi) |
| Maximum consumption flow | 10 l/min [2,6 US gpm) |
| Max permissible at hook up interface connection, leak oil @max leak oil flow | 2 bar (29 psi) |

Table 14-2 - Typical utility and interface requirements for pneumatic power supplied by the customer

| Description | Technical data |
| --- | --- |
| Max pneumatic supply pressure | 8 bar (116 psi) |
| Min pneumatic supply pressure | 6 bar (87 psi) |
| Compressed air purity class | ISO 8573-1:2010 (3: 2: 3) |
| Max air consumption | 7NL/s (15 SCFM) |

## Operational Temperature

Although the equipment is structurally and mechanically designed for operation at low temperatures as stated in chapter 8, a pre-heating  of the fluid system (typically done by circulation of the fluid and/or operating idle functions) may be required to ensure optimum viscosity and performance.

Heating devices or heat tracing may be supplied for some systems as an option in order to reduce or eliminate the warm-up time.

## Hydraulic Fluid for Fluid Power Systems

The equipment is designed for operation with mineral oil conforming to ISO 6743-4/ISO-L-HM or ISO-L-HV. Specific details are given in the user manual.

If environmentally acceptable fluid is preferred by the customer, HMH may adapt the fluid power system to comply with synthetic ester base fluid ISO 6743/ISO-L-HEES as an option.

## General Hydraulic Piping Systems

Small bore tubing systems (maximum diameter 42 mm) are corresponding with  the ISO 8434-1 connector standard.  Tubing and connectors  Material:  Stainless Steel.

Flexible rubber hoses and hose assemblies utilized in HMH fluid power systems are type approved by ABS and DNV for offshore and marine environment in hydraulic services at rated pressure.

For fluid power piping of larger sizes, see chapter 13 Piping.  Mud hoses according to API 7K may be used for  fluid power purposes in systems requiring  conductor diameters greater than 51 mm.

## Low Pressure Pneumatic Piping Systems

In protected environments like inside valve cabinets, polyurethane and polyamide tubing are used. In outdoor applications UV resistant black polyamide is used.

If additional resistance against frictional wear and tear is required, armored hosed or stainless steel tubing is used.

## Hydraulic Fluid for Passive Heave Compensator Systems

In order to avoid compression ignition, the passive heave compensators are designed for water-glycol based fluids conforming to MIL-H-22072C or DEF Stan 91-110

This includes the following equipment:

Riser tensioner systems

Drill string compensators, passive part only

Guideline and pod line compensators

# Electric Instrumentation and Power Systems

## General

Electrical installation within the equipment is performed according to IEC 61892-6, and IEC 60204-1 as applicable

If the above-mentioned electrical installation is located in hazardeous area, IEC 61892-7 and IEC 60079-14 are added.

Electric equipment, instruments and components located in hazardous area: Ref. Chapter 7.6 Hazardous Area Classification

Tagging of components and wires inside cabinets and junction boxes according to HMH’s SF01 - Coding Manual, that is based IEC 81346-2

Ingress protection (IP) is according to table 15-1. Degrees of protection are defined in IEC 61892-1 .

Table 15-1 - Standard protection degree for drilling equipment

| Equipment | Minimum protection degree |
| --- | --- |
| Outdoor installed field equipment | IP56 |
| Outdoor motors, general | IP56 |
| Outdoor motor in Draw work and Top Drive with forced ventilation | IP44 |
| Indoor motors | IP44 |
| Indoor equipment (depending on zone) | IP44 |
| Control cabinet in Local Instrument Room (LIR) | IP23 |
| Cabinets in weather protected field area | IP55 |

## Electrical Cabinets

The electrical cabinets will contain PLC’s, servers, remote I/O’s, network, power, and field termination. All cabinet assemblies are performed according to internal HMH workmanship manual.

Each control cabinet can be powered by one of these alternatives:

AC Power with one incoming feeder

AC Power with two incoming feeders connected to contactor, or via redundancy power supplies/modules

## Cables

Field cables used in the equipment manufactured by HMH are:

Flame retardant according to applicable parts of IEC 60332

Fire resistant when required by rules and function according to applicable parts of IEC 60331

Resistant to oil and drilling fluids according to NEK TS 606

Armored or braided, as selected by HMH.

Low smoke and halogen free

Cables where the above-mentioned specifications can't be met (e.g., flying lead at proximity switches), will have short lengths and be terminate in a nearby junction box. For added protection, these cables will be placed in hydraulic hoses or stainless-steel pipes

HMH is responsible for the cables within the equipment only, unless otherwise detailed in the respective product specification SA04. (Some equipment may for practical reasons be supplied with cable for connection to nearby interface point. Details are given in SA04 if applicable)

## Cable Entry and Gland

Cable entries and electrical interface points will be clearly indicated on the respective equipment general arrangements (GA)

Cable glands are arranged on the bottom or on the side walls of the junction box. When required, Multi Cable Transit (MCT) is used.

Ex e type cable glands are delivered as standard for hazardous areas. Ex d glands are used as required for the application.

Run-through type cable glands are standard delivery from HMH

EMC glands are used when applicable according to rules and regulations.

Glands for customer’s cables shall be supplied by customer.

## Junction Box

The junction box design enables the cable(s) to be taken from the entry point(s) to the connections in such a way that chafing against connections or other parts is obviated. The terminal bar and block assembly are designed to avoid impairment by the insulation shrinkage on the effectiveness of the electrical connections.

The junction box is made of stainless steel 316 material or GRP, certified as Ex e for hazardous area when applicable. The degree of protection grade is IP 56.

The junction box will be completely supplied with sufficient earth stud or PE/IE/ISE terminal for connection of cable’s sheet, armor or earth conductor to meet the requirements for earthing of the equipment.

## Thermistors

VFD controlled motors will be fitted with thermistors or other temperature detecting devices embedded in the stator windings from vendor. It is a safety device to prevent overheating.

Other motors will be delivered according to requirements and specifications.

## Earth

The protective earth (PE) will be connected to the ground at both ends using the power cable.

This is normally done with cable armour and through gland. If installation requires armour to be terminated in gland (i.e Exd gland), the cable will be fitted with insulated earth conductor. PE conductor will not be terminated in gland for equipment supplied with voltage above 50V

Instrumentation earth (IE/ISE) will be floating in the field and earthed in the cabinet

IE and/or ISE bus bars in junction boxes and cabinets are mounted on isolators.

Screens for Profibus cables and cables on certain VFD driven equipment will be earthed in both ends

Skids will be equipped with minimum two earth bosses, located on diagonally opposite corners welded to the skid main structure. Each earth boss is drilled for M10 bolt.

## Motor Heating

All outdoor motors with a rated power of 15 kW or above are fitted with internal heater elements in the windings.

# Control systems, HMI and networks

## General

HMH delivers complete, integrated control and monitoring systems for drilling and well intervention.

HMH’s control systems are designed to ensure safe, efficient and reliable working conditions and operations

The overall functionality is to manage the flow of control and information within the drilling system and with the operator.

Control systems from HMH are open for interconnection internally and with other vendors

Rig Safety and Automation System (SAS) is not part of the HMH offering.

Compliance to DNV-OS-D203 ISDS or ABS ISQM can be delivered as an option

A comprehensive service interface (HMI) for expected maintenance and fault finding is included for all control systems.

## Hardware

HMI systems, data logging, analysis and systems for advanced calculations are running on industrial Windows compatible computers.

Programmable Logic Controllers (PLC) are of the type Siemens Simatic S7, conforming to IEC 61131-3

I/O modules are connected to CPU’s through fieldbus, enabling cost efficient control system upgrades in the future.

## Network Design

An installation contains two main Ethernet networks

  - The PLC network interconnects PLCs. This includes connecting equipment control systems to operator chair and anti-collision.

  - The DrillView network connects HMI system and equipment control systems.

Important network signals have redundant communication lines.

I/O fieldbuses are based on Profibus and/or Profinet according to IEC 61158

For all network / fieldbus interfaces where HMH do not have control of cable routing, fiber optical cabling must be used.

## Operator Environment and Human Machine Interface (HMI)

DrillView is an open HMI system for distribution, monitoring and control of drilling and process data (real time and stored)

The DrillView alarm system is in compliance with the principles of IEC 62682 and EEMUA 191.

Different operator environments can be provided:

  - DrillView clients – industrial or office computers with high definition displays

  - Operator chairs - with touch panels, joysticks, and mouse for PC interface

  - Panels - radio panels, hydraulic panels, electrical panels, touch panels

The operator environment supports different modes of operation, from purely manual to highly automated and robotic control.

## Open Interfaces

HMH systems can interface a wide range of Variable Frequency Drives (VFD), motor starters and information management systems (IMS) upon request.

HMH HMI and control system can interface any 3rd party equipment like wireline, cement systems or other equipment.

HMH supplied equipment can interface any 3rd party HMI system upon request

## Built in Capabilities

HMH equipment and process control systems have built in capabilities for connection to our comprehensive portfolio of rig automation, advanced support systems and services.

## Cyber security

HMH’s cyber security program is based on IEC-62443-3-3 and  IEC 62443-2-4.

Security Level 1 is standard for our cyber security product offering. Higher security levels can be delivered upon request.

# Testing

Within the scope of this standard HMH equipment is typically subjected to the following tests:

Manufacturing tests and inspection of components and sub-assemblies

Assembly test and inspection. (Various early functional trials in workshop.)

Mechanical completion (MC inspection)

Internal acceptance test, including mechanical, electric, fluid power, control system, safety functions, MC check out and load tests as applicable for lifting equipment

Factory acceptance test

If the equipment cannot be fully functional tested before final installation, test plans are developed for each sub system to be tested as far as practical in the workshop.

HMH will supply procedures for installation and testing on board, and can also provide test personnel upon request(Test execution/assistance by HMH during/after installation is optional, and must be specified in contract/purchase order).

# Documentation and Marking of Equipment

## Standards for Documentation

The documentation is based on the NORSOK Z-018.

The standard file format for documentation is electronic pdf.

Coding of documentation is according to the HMH Coding Manual

## Supplier Master Document List (SMDL)

Document deliverables by HMH shall be highlighted in a supplier master document list (SMDL), to be issued by HMH within two weeks after order.

The content reflect HMH’ s product standardization, and will depend on product type, with respect to scope and disciplines involved.

Documents that contain relevant interface information are issued for customer’s review. Customer-HMH turnaround time on documents shall not exceed 10 working days. Upon exceeding said 10 days HMH may code the documents as appropriate.

Other documents, not relevant for interface, are issued for information only.

Typical content is shown in table 18-1

Table 18-1 - SMDL for drilling equipment – typical content

| Description of document | Issued for |
| --- | --- |
| Common for all equipment |  |
| Factory Acceptance Test (FAT) Procedure | Information |
| Commissioning Procedure | Information |
| Supplier Master Document List (SMDL) | Review |
| Dispatch Dossier (Incl. transport handling and preservation documents) | Information |
| Product Certificate issued by DNV GL or ABS, when applicable | Information |
| General Arrangement Drawing | Review |
| Spare Parts | Information |
| Additional for all equipment, except riser and drilling fluid processing. |  |
| Installation, Operation and Maintenance (IOM) Manual | Information |
| External Layout Drawings(E/I) | Review |
| Main Block Diagrams (E/I) | Review |
| Wiring Diagram (Interconnection) (E/I) | Review |
| Drawings, as necessary for operation and maintenance. | Information |
| Additional for Drilling fluid processing equipment |  |
| Pipe and Instrument Diagram(P&ID) | Review |
| Installation, Operation and Maintenance (IOM) Manual | Information |
| External Layout Drawings (E/I) | Review |
| Main Block Diagrams (E/I) | Review |
| Wiring Diagram (Interconnection) (E/I) | Review |
| Drawings, as necessary for operation and maintenance | Information |
| Additional for Drilling riser equipment |  |
| Design Premises | Information |
| Final Riser Analysis (DVR) | Information |
| Operation and Maintenance Manual (OMM) | Information |
| Handling and Lifting Procedure | Review |
| Manufacturing Record Book | Information |

## Manufacturing Records

Manufacturing records (Manufacturing Record Book, MRB) is produced and recorded in parallel with the production of a product designed and delivered by HMH

The records are used as part of a verification process, by the HMH quality surveyors, or by the classification society, when applicable

It will contain the manufacturer’s compiled records for the components, elements, and qualifications used during the fabrication of specific equipment

The MRB is filed electronically in the HMH documentation archive system

After finalizing production, the MRB is retained for a minimum of 10 years

The MRB will normally not be distributed to customers, except as noted for drilling riser.

Customers may, upon request and agreement, view the document at HMH or subcontactor’s premises.

A customer data book comprising a summary of MRB can be supplied as an option, for information.

## Documentation for Installation, Operation and Maintenance (IOM)

HMH will regard the equipment within the scope of this specification as equipment supplies, and the IOM is defined as such, (as opposed to engineering supplies)

Documentation related to plant engineering, except as necessary for interface, is outside the scope of this specification

IOM shall contain the documentation necessary to install, operate, maintain, and preserve the equipment in a safe manner

The NORSOK Z-018 is used as a foundation for the IOM.

The IOM Manual is a standard text document, and the drawings are contained in a separate package.

IOM is defined on the supplier master document list (SMDL).

IOM is issued for information only

IOM is filed electronically in the HMH archive system, where it is retained for a minimum of 10 years after finalizing the production of the equipment.

## Identification of Delivered Products

### Coding of Equipment Modules (Tag Numbering)

Equipment modules and components are identified in accordance with the ISO/IEC 81346 standard, as necessary for operation and maintenance, as detailed in HMH’s code manual.  The identification codes are labelled on the physical deliveries and referred to in the IOM documentation as applicable.

### Facility Tagging

A facility tagging system, which identifies equipment uniquely across the facility, is not part of HMH’s standard documentation.

Facility tagging ispart of the plant design, beyond the scope of this specification.

Upon request and agreement HMH may, however, mark the equipment with an additional facility tag number provided by the customer, and add a special front page to the standard IOM document.

Customer’s main tag number is noted on the front page, but is not referred to elsewhere in the document.

# Appendix A – Reference Documents

The complete list of all standard references given in this document is presented in the table below. The standards apply as described in the text of this document. (Note that some are optional).

Table 19-1 - Reference documents

| Document code | Title |
| --- | --- |
| ABS: CDS Guide | Guide for Classification of Drilling Systems |
| ABS: ISQM Guide | Guide for Integrated Software Quality Management |
| ABS: MODU | Mobile Offshore Drilling Units, Part 4- Machinery and Systems |
| API Bulletin 2INT-MET | Interim Guidance on Hurricane Conditions in the Gulf of Mexico |
| API RP 16Q | Recommended Practice for Design, Selection, Operation and Maintenance of Marine Driller Rising Systems |
| API RP 17B | Flexible Pipe, Design and operation of subsea production systems |
| API Spec 16A | Drill-Through Equipment |
| API Spec 16C | Choke and Kill System |
| API Spec 16F | Marine Drilling Riser Equipment |
| API Spec 16R | Marine Drilling Riser Couplings |
| API Spec 17D | Design and Operation of Subsea Production Systems |
| API Spec 2RD | Dynamic Risers for Floating Production Systems |
| API Spec 4F | Drilling and well servicing structures |
| API Spec 5L | Specification for Line Pipe |
| API Spec 6A | Wellhead and Christmas Tree Equipment |
| API Spec 7K | Drilling Equipment |
| API Spec 8C | Drilling and Production Hoisting Equipment |
| API STD 53 | Blowout Prevention Equipment Systems for Drilling Wells |
| ASME B31.3 | Process Piping. |
| ASME VIII BPVC | ASME Boiler and Pressure Vessel Code |
| ATEX 2014/34/EU 2014/34/EU | ATEX Directive |
| DEF Stan 91-110 | Defense standard. Wear performance test for aqueous polyglycol based hydraulic fluids |
| DNV-OS-D203 | Integrated Software Dependent Systems |
| DNV-OS-E101 | Drilling Facilities |
| DNV-OS-F201 | Dynamic Risers - DNV Offshore Standards |
| DNV-RP-C201 | Buckling strength of plated structures |
| DNV-RP-C203 | Fatigue Design of Offshore Structures |
| DNV-RP-C205 | Environmental Conditions and Environmental Loads |
| DNV-ST-0378 | Standard for offshore and platform lifting appliances |
| DROPS | Dropped Objects Awareness and Prevention (DROPS) handbook, “Reliable Securing” Guidance. Revision 03 |
| EEMUA 191 | Alarm systems - a guide to design, management and procurement |
| EMC 2014/30/EU 2004/108/EC | Electromagnetic Compatibility Directive (EMC) |
| EN 1127-1 | Explosive atmospheres - Explosion prevention and protection - Part 1: Basic concepts and methodology |
| EN 13586 | Access – Cranes |
| EN 1993-1 | Eurocode 3- Structural Design |
| FEM | Rules for the design of hoisting appliances |
| FME | Regulation for maritime electrical installation- Directorate of civil protection, Norway |
| IEC 60079 | Explosive atmospheres. Equipment. (Relevant parts: 1,7,10,11,14 and15) |
| IEC 60204-1 | Safety of machinery-Electrical equipment of machines |
| IEC 60300 | Dependability management part 3-11 - Application guide RCM |
| IEC 60331 | Tests for electric cables under fire conditions - Circuit integrity |
| IEC 60332 | Tests on electric and optical fiber cables under fire conditions. |
| IEC 60445 | Identification of equipment terminals, conductor terminations and conductors |
| IEC 60529 | Degrees of protection provided by enclosures |
| IEC 61131-3 | Programmable controllers. Part 3: Programming languages |
| IEC 61158 | Industrial communications networks. Fieldbus specification |
| IEC 61892 | Mobile and fixed offshore units. Electrical installations |
| IEC 62443 | Industrial communication networks-Network and systems security |
| IEC 62682 | Management of alarms systems for the process industries |
| ISO 10423 | Drilling and prod. equipment - Wellhead and Christmas Tree Equipment |
| ISO 11064 | Ergonomic design of control centers, Part 1–3 |
| ISO 1179-1 | Connections for general use and fluid power –Part 1: Threaded ports |
| ISO 12100 | Safety of machinery – General principles for design. Risk assessment and risk reduction |
| ISO 1219-1 | Fluid power systems and components - graphical symbols and circuit diagrams - Graphical symbols for conventional use and data processing applications |
| ISO 1219-2 | Fluid power systems and components- graphical symbols and circuit diagrams- Circuit diagrams |
| ISO 12944-5 | Paints and varnishes. Corrosion protection for steel structures by protective paint. systems. Part 5: Protective paint systems |
| ISO 12944-9 | Protective paint systems and laboratory test methods for offshore related structures |
| ISO 13533 | Drilling and production equipment -- Drill-through equipment |
| ISO 13625 | Marine Drilling Riser Couplings |
| ISO 13628-11 | Subsea Wellhead and Tree Equipment – Part 11: Flexible pipe systems for subsea and marine applications |
| ISO 13849-1 | Safety of machinery — Safety-related parts of control systems, Part 1: General principles for design |
| ISO 13850 | Safety of machinery — Emergency stop function — Principles for design |
| ISO 14122 | Safety of machinery – Permanent means of access to machinery |
| ISO 1461 | Hot dip galvanized coatings on fabricated iron and steel articles. Specifications and methods |
| ISO 3183 | Steel pipe for pipe line transportation |
| ISO 3746 | Acoustics – Determinations of sound power levels and sound energy levels of noise sources using sound pressure – Survey method using an enveloping measurement surface over a reflecting plane |
| ISO 4406 | Hydraulic fluid power - Fluids - Method for coding the level of contamination by solid particles |
| ISO 4413 | Hydraulic fluid power — General rules and safety requirements for systems and their components |
| ISO 4414 | Pneumatic fluid power — General rules and safety requirements for systems and their components |
| ISO 6743-4/ISO-L-HM | Lubricants, industrial oils and related products (class L) Part 4: Family H (Hydraulic systems) |
| ISO 80079 | Non electrical equipment for explosive atmospheres, (part 36 and 37) |
| ISO 8434-1 | Metallic tube connections for fluid power and general use Part 1: 24° cone connectors |
| ISO 8573-1 | Compressed air. Part 1: Contaminants and purity classes |
| ISO 9614-2 | Acoustics – Determination of Sound Power level and Noise Source Using Sound Intensity – Part 2: Measurement by Scanning |
| ISO/IEC 81346 | Industrial systems, installations and equipment and industrial products — Structuring principles and reference designations Part 1: Basic rules |
| LVD 2014/35/EU | Low Voltage Directive |
| MD 2006/42/EC | Machinery Directive 2006/42/EC |
| MIL-H-22 72C | Military specification. Hydraulic fluid, catapult. NATO code no. H-579 |
| NACE MR 0175 ISO 15156 | Petroleum and natural gas industries. Materials for use in H2S-containing Environments in oil and gas production |
| NEK HD 308 S2 | Identification of cores in cables and flexible cords |
| NEK TS 606 | Cables for offshore installations |
| NORSOK D-001 | Drilling Facilities |
| NORSOK R-002 | Lifting Equipment |
| NORSOK S-002 | Working Environment |
| NORSOK Z-006 | Preservation |
| NORSOK Z-018 | Supplier’s documentation of equipment |
| PED 2014 /68/EU | Pressure Equipment Directive 2014/68/EU (PED) |
| PSA | Facilities Regulations |
| SAE JA1011 | Evaluation Criteria for RCM Process |
| SAE JA1012 | A Guide to the Reliability Centered Maintenance (RCM) Standard |
| SFS Handbook. Ed.3 | Best Practice – Dropped Object Prevention. Working Together for Safety. |